OBJECTIVE: FoxP3 procedure for PBMC and purified cells

For optimum FoxP3+ staining results:

- Use the 259D clone conjugated to Alexa-Fluor 647. Use a volume of 10 μL/test.

- Incubate ab longer than usual (increased signals between 30, 45 & 60 min.)

- It is critical to use a detergent-free PBS buffer for the first wash.

- R3 solution provided with the kit must be used for the second wash step.

BC PerFix-nc buffer protocol - less actual hands-on time ; few centrifugations limit cell loss during washes

SPECIMEN: Whole blood collected in an anticoagulant such as ACD, EDTA or heparin.

REAGENTS

PerFix-nc Kit - PN B10825 - 75 tests; store @ RT, 12-month shelf life at manufacturing

PerFix-nc Buffer 1: Fixative Reagent

PerFix-nc Buffer 2: Permeabilizing Reagent

PerFix-nc Buffer 3: Final 10X Solution (in diH2O)

1X PBS

STAINING

Modifications to the protocol for FoxP3 staining in PBMC (written in green)

Total processing time is < 105 min. Total hands-on is < 30 min.

- Resuspend PBMC in 100% FCS (or FBS) @ 5 M/mL up to 40 M/mL (= 25K/50uL to 2M/50uL).

- Pipet (as in WB procedure) 50 μL of these cells into bottom of each appropriately labeled tube and add 2.5uL of buffer R1 Fixative Reagent (=HALF the WB volume!).

- Vortex immediately, incubate for 15 min. at room temperature (18 – 25°C).

- Vortex the fixed cells and add 150 μL R2 Permeabiling reagent (=HALF the WB ).

- Immediately add the intracellular and gating markers (preferably, antibodies are pre-mixed with the R3 Permeabilizing Reagent and added at the end of the fixation step). NB: Incubate surface and intra markers simultaneously during perm step

- Vortex ASAP, incubate for 60 min at RT for FOX-P3.

- Perform an additional wash step with 3 mL PBS, incubate for 5 min at RT.

- Spin 500 g for 5 min, aspirate

- Perform the second wash with 1.5 – 3mL of 1X R3 Reagent, vortex ASAP.

- Centrifuge (500 g for 5 min.), aspirate & resuspend in 500 μL of the1X R3 Reagent

Notes: a) scale up PBMC and BC buffer volume proportionally; b) PC7, APC-Cy7, ECD tandems compatible with this buffer as tested by Beckman Coulter; c for ZAP-70: incubation 45 min optimal; d) for regular PBMC cytokine ICS: incubate 15min RT in the dark;              e) fluorochrome-conjugated antibodies against intracellular epitopes and surface molecules are added together during perm step

Ref.: Beckman Coulter FoxP3 staining in human whole blood using PerFix-nc (optimized protocol) by Fabrice Malergue PhD (1), Carlos A. Garcia PhD (2) and Tewfik Miloud PhD (1) (1) Global Assay and Applications Development, Beckman Coulter Life Science Marseille France; (2) Life Science Research Beckman Coulter Life Science Miami USA